AC1200 Wireless Gigabit Ceiling Mount Access Point

EAP912

OVERVIEW：

Stonet EAP912 is designed for corporate offices, hotels, supermarkets and other scenario design, can meet the needs of a large number of wireless clients. Full gigabit port, 1200Mbps wireless dual band; 4 built-in high gain antennas. 128M memory, stable standby 60 units. The power is linearly adjustable to effectively reduce the interference before the AP. 48V standard POE power supply +DC12V/1A, two power supply modes, flexible networking.

FEATURES：

• Simultaneous 300Mbps on 2.4GHz and 867Mbps on 5GHz

• Adjustable power, reduce the interference between the AP

• Automatically weed out weak signal and improve wireless network performance

• Power adjustable to reduce interference

• Unified AP management without the need for additional AC controllers

• Support two power supply mode, more flexible networking

SPECIFICATION：

| Hardware |  |
| --- | --- |
| Interface | 1 * 10/100/1000M adaptive LAN port(POE power supply) 1 * 10/100/1000M adaptive WAN port |
| Antenna | 4 * built-in high gain antenna |
| Button | Default Button |
| Power Supply | 48V standard POE or external DC 12V/1A power supply |
| LED | 6 * red and blue LED |
| Mounting | Ceiling /Wall Mounting (Kits included) |
| Software |  |
| Memory | 128MB |
| FLASH | 16MB |
| Forwarding rate | 10/100/1000Mbps |
| Wireless |  |
| Frequency | 2.4 GHz and 5 GHz |
| Signal Rate | 300Mbps at 2.4GHz, 867Mbps at 5GHz |
| Wireless Standards | IEEE802.11b/g/n/ac |
| Features | Wireless terminal Wireless network Intranet Settings Signal conditioning Host Black and white list Internet access Settings (routing mode/Bridge mode) Advanced Settings Port mapping Dynamic domain names(dyndns/noip/oray) Managing Passwords Time setting Initialization (Default restore) Schedule tasks (periodic restarts) Firmware upgrade (manual upgrade/online upgrade /AC upgrade) Dial-up log Device information Indicator light |
| Others |  |
| Dimensions | 145 * 145 * 45mm |
| Environment | Operating Temperature : 0℃ ~ 40℃ Operating Humidity : 10% ~ 90% RH non condensing Storage Temperature : -20℃ ~ 70℃ Storage Humidity : 5% ~ 90% RH non condensing |