8GE POE+2GE Uplink+2SFP            Switch

P112GS

OVERVIEW：

Stonet P112GS provides 8 10/100/1000M adaptive RJ45 ports,2 1000M adaptive RJ45 cascaded ports and 2 SFP Gigabit ports; Ports 1 to 8 support standard POE power supply, and the ports can automatically detect and identify the power device that conforms to the IEEE 802.3AT/AF standard and supply power to it through the network cable, which can be used as an Ethernet power supply device.Not only can rack, desktop multi-way installation, but also has AI+VLAN; AI Monitor: Automatically detect and handle network anomalies in front-end devices (AI watchdog).

FEATURES：

• 8 10/100/1000M adaptive RJ45 ports,2 1000M adaptive RJ45 cascaded ports,2 SFP Gigabit ports

• Follow IEEE 802.3 Ethernet and IEEE802.3u Fast Ethernet Protocol

• POE IEEE 802.3af/at compliant Powered Device (PD)

• Supporting normal switch/automatic AI detection + port isolation (combined function) second mode

• Auto MDI/MDIX crossover for all ports

• Support full duplex based on IEEE802.3x and half duplex based on Backpressure

• All ports have wire-speed forwarding capability

• MAC address auto-learning & auto-aging

• Power supply: built-in 120W

SPECIFICATION：

| Hardware |  |
| --- | --- |
| Interface | 8*1000Mbps POE RJ45 ports + 2*Gigabit cascaded RJ45 ports + 2*Gigabit SFP ports |
| LED | PWR - Lighting:The product is powered on Function indication - Lighting:The function switch is on and AI+VLAN port isolation is enabled Port lights - Lighting:Port link is normal,Blinking: Data in transit |
| Backplane bandwidth | 24G |
| MAC Address Table Size | 2K |
| Cabling type | 1000Base-T: 4 pairs of Cat5e or above UTP/STP(≤100m) 100Base-TX: 2 pairs of class 5 (Cat5) or more UTP/STP(≤100m) 10Base-TX: 2 pairs of class 5 (Cat6) or above UTP/STP(≤250m) |
| Power supply | AC input 100-240V, standard 120W power supply |
| Software |  |
| Basic function | Follow IEEE 802.3 Ethernet and IEEE802.3u Fast Ethernet Protocol Supports POE IEEE 802. 3af/at compliant Powered Device (PD) Store-and-forward switching scheme Compatible with certified non-standard electrified equipment (PD) Support IEEE802.3X full duplex and Backpressure half duplex MAC address auto-learning & auto-aging VLAN port isolation AI Monitor |
| Standards | IEEE 802.3 10BASE-T Ethernet IEEE 802.3i 10BASE-T Ethernet IEEE 802.3u 100BASE-TX Fast Ethernet IEEE 802.3ab 1000BASE-T Gigabit Ethernet IEEE 802.3af/at, Power over Ethernet IEEE 802.3x Flow Control |
| Mode | Normal (default mode) VLAN (AI+ port isolation) |
| Data Transfer Rate | 10Mbps(Half-duplex and Full-duplex) 100Mbps(Half-duplex and Full-duplex) 1000Mbps(Full-duplex) |
| Others |  |
| Environment | Operation Temperature：0℃～40℃ Storage Temperature：-40℃～70℃ Operation Humidity：10%～90% RH Non-condensing Storage Humidity：5%～90% RH Non-condensing |
| Dimensions | 190*140*43mm(L x W x H ) |