SPT-P13TG-LR(S)

10Gbps SFP+ Optical Transceiver, 10km or 20km Reach

Features

- Optical interface compliant to IEEE 802.3ae 10GBASE-LR

- Electrical interface compliant to SFF-8431

- Hot Pluggable

- 1310nm DFB transmitter, PIN photo-detector

- Operating case temperature: Standard:	0 to +70°C Industrial: -40 to +85°C

- Low power consumption

- Applicable for 10km or 20km SMF connection

- All-metal housing for superior EMI performance

- Advanced firmware allow customer system encryption information to be stored in transceiver

- Cost effective SFP+ solution, enables higher port densities and greater bandwidth

Applications

- 10GBASE-LR/LW Ethernet

- SONET OC-192 / SDH

- 10G Fibre Channel

Product Description

This 1310 nm 10G SFP+ LR transceiver is a very compact 10Gb/s optical transceiver module for serial optical communication applications at 10Gb/s. The SFP+LR converts a 10Gb/s serial electrical data stream to 10Gb/s optical output signal and a 10Gb/s optical input signal to 10Gb/s serial electrical data streams. The high speed 10Gb/s electrical interface is fully compliant with SFI specification.

The high performance 1310nm DFB transmitter and high sensitivity PIN receiver provide superior performance for Ethernet applications at up to 10km or 20km links

The SFP+ Module compliants with SFF-8431, SFF-8432 and IEEE 802.3ae 10GBASE-LR. Digital diagnostics functions are available via a 2-wire serial interface, as specified in SFF-8472.

The fully SFP compliant form factor provides hot pluggability, easy optical port upgrades and low EMI emission.

Absolute Maximum Ratings

These values represent the damage threshold of the module.	Stress in excess of any of the individual

Absolute Maximum Ratings can cause immediate catastrophic damage to the module even if all other parameters are within Recommended Operating Conditions.

| Parameter | Symbol | Min. | Typical | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| Storage Temperature | TS | -40 |  | +85 | °C |
| Case Operating Temperature | TA | 0 |  | 70 | °C |
| Maximum Supply Voltage | Vcc | -0.5 |  | 4 | V |
| Relative Humidity | RH | 0 |  | 85 | % |

Electrical Characteristics (TOP = 0 to 70 °C, VCC = 3.135 to 3.465 )

| Parameter | Symbol | Min. | Typical | Max. | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | 3.135 |  | 3.465 | V |  |
| Supply Current | Icc |  |  | 300 | mA |  |
| Power Consumption | P |  |  | 1.5 | W |  |
| Transmitter Section: |  |  |  |  |  |  |
| Input differential impedance | Rin |  | 100 |  | Ω | 1 |
| Tx Input Single Ended DC Voltage Tolerance (Ref VeeT) | V | -0.3 |  | 4 | V |  |
| Differential input voltage swing | Vin,pp | 180 |  | 700 | mV | 2 |
| Transmit Disable Voltage | VD | 2 |  | Vcc | V | 3 |
| Transmit Enable Voltage | VEN | Vee |  | Vee+0. 8 | V |  |
| Receiver Section: |  |  |  |  |  |  |
| Single Ended Output Voltage Tolerance | V | -0.3 |  | 4 | V |  |
| Rx Output Diff Voltage | Vo | 180 |  | 850 | mV |  |
| Rx Output Rise and Fall Time | Tr/Tf | 30 |  |  | ps | 4 |
| LOS Fault | VLOS fault | 2 |  | VccHOS T | V | 5 |
| LOS Normal | VLOS norm | Vee |  | Vee+0. 8 | V | 5 |

Note:

- Connected directly to TX data input pins. AC coupling from pins into laser driver IC.

- Per SFF-8431 Rev 3.0

- Into 100 ohms differential termination. 4.20%～80%

5.LOS is an open collector output. Should be pulled up with 4.7k – 10kΩ on the host board. Normal operation is logic 0; loss of signal is logic 1. Maximum pull-up voltage is 5.5V.

Optical Parameters (TOP = 0 to 70 °C, VCC = 3.135 to 3.465 )

| Parameter | Symbol | Min. | Typical | Max. | Unit | Note |
| --- | --- | --- | --- | --- | --- | --- |

| Transmitter Section: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Center Wavelength | λt | 1290 | 1310 | 1330 | nm |  |
| spectral width | △λ |  |  | 1 | nm |  |
| Average Optical Power | Pavg | -8.2 |  | 0.5 | dBm | 1 |
| Optical Power OMA | Poma | -5.2 |  |  | dBm |  |
| Laser Off Power | Poff |  |  | -30 | dBm |  |
| Extinction Ratio | ER | 3.5 |  |  | dB | 7 |
| Extinction Ratio | ER | 6 |  |  | dB | 6 |
| Transmitter Dispersion Penalty | TDP |  |  | 3.2 | dB | 2 |
| Relative Intensity Noise | Rin |  |  | -128 | dB/Hz | 3 |
| Optical Return Loss Tolerance |  | 20 |  |  | dB |  |
| Receiver Section: |  |  |  |  |  |  |
| Center Wavelength | λr | 1260 |  | 1355 | nm |  |
| Receiver Sensitivity | Sen |  |  | -14.5 | dBm | 4,7 |
| Receiver Sensitivity | Sen |  |  | -14.5 | dBm | 4,6 |
| Stressed Sensitivity (OMA) | SenST |  |  | -10.3 | dBm | 4 |
| Los Assert | LOSA | -25 |  | - | dBm |  |
| Los Dessert | LOSD |  |  | -15 | dBm |  |
| Los Hysteresis | LOSH | 0.5 |  |  | dB |  |
| Overload | Sat | 0 |  |  | dBm | 5 |
| Receiver Reflectance | Rrx |  |  | -12 | dB |  |

Note:

- Average power figures are informative only, per IEEE802.3ae.

- TWDP figure requires the host board to be SFF-8431compliant. TWDP is calculated using the Matlab code provided in clause 68.6.6.2 of IEEE802.3ae.

- 12dB reflection.

- Conditions of stressed receiver tests per IEEE802.3ae. CSRS testing requires the host board to be SFF-8431 compliant.

- Receiver overload specified in OMA and under the worst comprehensive stressed condition.

- SONET OC-192 / SDH

- 10GBASE-LR/LW Ethernet

Timing Characteristics

| Parameter | Symbol | Min. | Typical | Max. | Unit |
| --- | --- | --- | --- | --- | --- |
| TX_Disable Assert Time | t_off |  |  | 10 | us |
| TX_Disable Negate Time | t_on |  |  | 1 | ms |
| Time to Initialize Include Reset of TX_FAULT | t_int |  |  | 300 | ms |
| TX_FAULT from Fault to Assertion | t_fault |  |  | 100 | us |
| TX_Disable Time to Start Reset | t_reset | 10 |  |  | us |
| Receiver Loss of Signal Assert Time | TA,RX_LOS |  |  | 100 | us |

| Receiver Loss of Signal Deassert Time | Td,RX_LOS |  |  | 100 | us |
| --- | --- | --- | --- | --- | --- |
| Rate-Select Chage Time | t_ratesel |  |  | 10 | us |
| Serial ID Clock Time | t_serial-cloc k |  |  | 100 | kHz |

Pin definition

The SFP+ modules are hot-pluggable. Hot pluggable refers to plugging in or unplugging a module while the host board is powered. The SFP+ host connector is a 0.8 mm pitch 20 position right angle improved connector specified by SFF-8083, or stacked connector with equivalent with equivalent electrical performance. Host PCB contact assignment is shown in Figure 2 and contact definitions are given in Table 2. SFP+ module contacts mates with the host in the order of ground, power, followed by signal as illustrated by Figure 3 and the contact sequence order listed in Table 2.

Interface to Host PCB

Module Contact Assignment

| Pin | Symbol | Name/Description |
| --- | --- | --- |
| 1 | VEET [1] | Transmitter Ground |
| 2 | Tx_FAULT [2] | Transmitter Fault |
| 3 | Tx_DIS [3] | Transmitter Disable. Laser output disabled on high or open |
| 4 | SDA [2] | 2-wire Serial Interface Data Line |
| 5 | SCL [2] | 2-wire Serial Interface Clock Line |
| 6 | MOD_ABS [4] | Module Absent. Grounded within the module |
| 7 | RS0 [5] | RS0 for Rate Select: Open or Low = Module supports ≤4.25Gbps High = Module supports 9.95 Gb/s to 10.3125 Gb/s |
| 8 | RX_LOS [2] | Loss of Signal indication. Logic 0 indicates normal operation |
| 9 | RS1 [5] | No connection required |
| 10 | VEER [1] | Receiver Ground |
| 11 | VEER [1] | Receiver Ground |
| 12 | RD- | Receiver Inverted DATA out. AC Coupled |
| 13 | RD+ | Receiver DATA out. AC Coupled |
| 14 | VEER [1] | Receiver Ground |
| 15 | VCCR | Receiver Power Supply |
| 16 | VCCT | Transmitter Power Supply |
| 17 | VEET [1] | Transmitter Ground |
| 18 | TD+ | Transmitter DATA in. AC Coupled |
| 19 | TD- | Transmitter Inverted DATA in. AC Coupled |
| 20 | VEET [1] | Transmitter Ground |

SFP+ Module PIN Definition

SFP Module EEPROM Information and Management

The SFP modules implement the 2-wire serial communication protocol as defined in the SFP -8472. The serial ID information of the SFP modules and Digital Diagnostic Monitor parameters can be accessed through the I2C interface at address A0h and A2h. The memory is mapped in Table 1. Detailed ID information (A0h) is listed in Table 2. And the DDM specification at address A2h. For more details of the memory map and byte definitions, please refer to the SFF-8472, “Digital Diagnostic Monitoring Interface for Optical Transceivers”. The DDM parameters have been internally calibrated.

Table 1. Digital Diagnostic Memory Map (Specific Data Field Descriptions)

Table 2 - EEPROM Serial ID Memory Contents (A0h)

| Data Address | Length (Byte) | Name of Length | Description and Contents |
| --- | --- | --- | --- |
| Base ID Fields |  |  |  |
| 0 | 1 | Identifier | Type of Serial transceiver (03h=SFP) |
| 1 | 1 | Reserved | Extended identifier of type serial transceiver (04h) |
| 2 | 1 | Connector | Code of optical connector type (07=LC) |
| 3-10 | 8 | Transceiver | 10G Base-LR |
| 11 | 1 | Encoding | 64B/66B |
| 12 | 1 | BR, Nominal | Nominal baud rate, unit of 100Mbps |
| 13-14 | 2 | Reserved | (0000h) |
| 15 | 1 | Length(9um) | Link length supported for 9/125um fiber, units of 100m |
| 16 | 1 | Length(50um) | Link length supported for 50/125um fiber, units of 10m |

| 17 | 1 | Length(62.5um) | Link length supported for 62.5/125um fiber, units of 10m |
| --- | --- | --- | --- |
| 18 | 1 | Length(Copper) | Link length supported for copper, units of meters |
| 19 | 1 | Reserved |  |
| 20-35 | 16 | Vendor Name | SFP vendor name: STONET |
| 36 | 1 | Reserved |  |
| 37-39 | 3 | Vendor OUI | SFP transceiver vendor OUI ID |
| 40-55 | 16 | Vendor PN | Part Number: (ASCII) |
| 56-59 | 4 | Vendor rev | Revision level for part number |
| 60-62 | 3 | Reserved |  |
| 63 | 1 | CCID | Least significant byte of sum of data in address 0-62 |
| Extended ID Fields |  |  |  |
| 64-65 | 2 | Option | Indicates which optical SFP signals are implemented (001Ah = LOS, TX_FAULT, TX_DISABLE all supported) |
| 66 | 1 | BR, max | Upper bit rate margin, units of % |
| 67 | 1 | BR, min | Lower bit rate margin, units of % |
| 68-83 | 16 | Vendor SN | Serial number (ASCII) |
| 84-91 | 8 | Date code | STONET’s Manufacturing date code |
| 92-94 | 3 | Reserved |  |
| 95 | 1 | CCEX | Check code for the extended ID Fields (addresses 64 to 94) |
| Vendor Specific ID Fields |  |  |  |
| 96-127 | 32 | Readable | STONET specific date, read only |
| 128-255 | 128 | Reserved | Reserved for SFF-8079 |

Digital Diagnostic Monitor Characteristics

| Data Address | Parameter | Accuracy | Unit |
| --- | --- | --- | --- |
| 96-97 | Transceiver Internal Temperature | ±3.0 | °C |
| 100-101 | Laser Bias Current | ±10 | % |
| 100-101 | Tx Output Power | ±3.0 | dBm |
| 100-101 | Rx Input Power | ±3.0 | dBm |
| 100-101 | VCC3 Internal Supply Voltage | ±3.0 | % |

Regulatory Compliance

The SFP+ complies with international Electromagnetic Compatibility (EMC) and international safety requirements and standards (see details in Table following).

| Electrostatic Discharge (ESD) to the Electrical Pins | MIL-STD-883E Method 3015.7 | Class 1(>1000 V) |
| --- | --- | --- |

| Electrostatic Discharge (ESD) to the Duplex LC Receptacle | IEC 61000-4-2 GR-1089-CORE | Compatible with standards |
| --- | --- | --- |
| Electromagnetic Interference (EMI) | FCC Part 15 Class B EN55022 Class B (CISPR 22B) VCCI Class B | Compatible with standards |
| Laser Eye Safety | FDA 21CFR 1040.10 and 1040.11 EN60950, EN (IEC) 60825-1,2 | Compatible with Class 1 laser product. |

Recommended Circuit

Recommended Host Board Power Supply Circuit

Recommended High-speed Interface Circuit

Unit:mm

ESD

This transceiver is specified as ESD threshold 1kV for high speed pins and 2kV for all other electrical input pins, tested per MIL-STD-883, Method 3015.4 /JESD22-A114-A (HBM). However, normal ESD precautions are still required during the handling of this module. This transceiver is shipped in ESD protective packaging. It should be removed from the packaging and handled only in an ESD protected environment.

ESD

This transceiver is specified as ESD threshold 1kV for high speed pins and 2kV for all other electrical input pins, tested per MIL-STD-883, Method 3015.4 /JESD22-A114-A (HBM). However, normal ESD precautions are still required during the handling of this module. This transceiver is shipped in ESD protective packaging. It should be removed from the packaging and handled only in an ESD protected environment.

| Part Number | Product Description |
| --- | --- |
| SPT-P13TG-LR | 10Gbps SFP+ Optical Transceiver, 10km Reach, 0ºC ~ +70ºC, DDM |